# Empirical Speedup Fitting, Micro-Architectural Bottlenecks, and Loop Scheduling in OpenMP

Student Full Name: Orynbay Ali
Student ID Number: 230103160
Institutional Email / Git URL: 230103160@sdu.edu.kz , https://github.com/AliOrynbay
Practicum Date & Time (Slot): 24 September 2026
Laptop Model & OS: Windows 11 / MSYS2 UCRT64
CPU Model & Generation: AMD Ryzen 5 5600H
Physical Cores: 6
Logical SMT Threads: 12
Target Workload Size N: 13,160,000
L1 / L2 Cache Line Size: 64 bytes

## Phase 1 — Hardware Audit

- CPU: AMD Ryzen 5 5600H

- Physical cores: 6

- Logical threads: 12

- L2 Cache: 3072 KB

- L3 Cache: 16384 KB

- Cache line size: 64 bytes

- OpenMP max threads: 12

## Phase 2 — Sequential Baseline

N = 13,160,000

| Run | Time (s) |
| --- | --- |
| Run 1 | 2.251 |
| Run 2 | 2.242 |
| Run 3 | 2.251 |

Run 1 was discarded as the warm-up run.

T_seq = (2.242 + 2.251) / 2 = 2.2465 s

Maximum stopping time: 688
Checksum: 80,605,186

## Phase 3 — OpenMP Scaling

| Threads | Run 1 | Run 2 | Run 3 | Average | Speedup |
| --- | --- | --- | --- | --- | --- |
| 2 | 1.148 | 1.151 | 1.148 | 1.1495 | 1.954 |
| 4 | 0.607 | 0.578 | 0.578 | 0.5780 | 3.887 |
| 8 | 0.362 | 0.354 | 0.372 | 0.3630 | 6.189 |
| 12 | 0.266 | 0.260 | 0.259 | 0.2595 | 8.657 |

The 1-thread run was 2.239 s and was not used as the sequential baseline.

Estimated parallel fraction: p = 97.66%

### Amdahl's Law

| Threads | Empirical Speedup | Amdahl Speedup | Reality Ratio |
| --- | --- | --- | --- |
| 2 | 1.954 | 1.954 | 1.000 |
| 4 | 3.887 | 3.738 | 1.040 |
| 8 | 6.189 | 6.875 | 0.900 |
| 12 | 8.657 | 9.546 | 0.907 |

## Phase 4A — False Sharing

Maximum physical thread count: 6

| Version | Run 1 | Run 2 | Run 3 | Average |
| --- | --- | --- | --- | --- |
| Naive | 0.427 | 0.471 | 0.410 | 0.4405 |
| Padded | 0.412 | 0.423 | 0.434 | 0.4285 |

Hits: 10,604,608

Naive throughput: 29.88 million values/s
Padded throughput: 30.71 million values/s
Penalty ratio: 1.028

The cache line size is 64 bytes. In the naive version, thread counters can share the same cache line, causing cache-coherence traffic. Padding separates counters into different cache lines and slightly improves performance.

## Phase 4B — OpenMP Scheduling

Maximum threads: 12

| Schedule | Run 1 | Run 2 | Run 3 | Average |
| --- | --- | --- | --- | --- |
| static | 0.266 | 0.262 | 0.256 | 0.2590 |
| static,1000 | 0.261 | 0.257 | 0.253 | 0.2550 |
| dynamic,100 | 0.254 | 0.249 | 0.245 | 0.2470 |
| dynamic,10000 | 0.249 | 0.255 | 0.242 | 0.2485 |
| guided | 0.245 | 0.244 | 0.248 | 0.2460 |

All runs produced checksum 80,605,186.

## Phase 5 — Analysis

Q1. Cache line and MESI:
The measured cache line size is 64 bytes. Several thread counters can fit into one cache line. When different threads update them, the cache line may move between cores because of cache coherence. Padding reduces this problem.

Q2. SMT:
The Ryzen 5 5600H has 6 physical cores and 12 logical threads using SMT. Two logical threads share resources of one physical core, so 12 threads do not provide the performance of 12 independent cores.

Q3. Amdahl's Law:
Amdahl's Law is a simplified theoretical model. Real performance is affected by OpenMP overhead, scheduling, cache behavior, SMT, CPU frequency, and operating-system activity. Therefore, measured speedup can differ from the theoretical value.

Q4. Scheduling:
Static scheduling has low overhead and works well when iterations have similar costs. Dynamic scheduling improves load balancing but adds overhead. Smaller chunks give better balancing but more scheduling overhead. Guided scheduling provides a compromise between the two.

## Conclusion

The experiment showed the difference between theoretical and real parallel performance. The sequential execution time was 2.2465 s, while the 12-thread execution time was 0.2595 s, giving an empirical speedup of 8.657×. The estimated parallel fraction was 97.66%. The false-sharing experiment showed a small improvement from padding, while different OpenMP scheduling strategies produced different execution times. Overall, CPU architecture, cache coherence, SMT, scheduling, and runtime overhead all affect real parallel performance.